### Q1. Circuit vs. Packet Switching

This question explores the difference between circuit-switched and packet-switched networks in terms of user capacity.

#### a) Maximum users with circuit switching

In circuit switching, each user gets a dedicated portion of the link's bandwidth for the entire duration of their connection, whether they are transmitting or not.

- Total Link Bandwidth: 5 Mbps = 5,000 kbps

- Bandwidth per User: 150 kbps

The maximum number of users is the total bandwidth divided by the bandwidth required per user.

Maximum Users=Bandwidth per UserTotal Bandwidth​=150 kbps5000 kbps​≈33.33

Since we can't have a fraction of a user, we take the floor of the result. Maximum number of users = 33

#### b) Remainder of the problem: Packet Switching

For the next parts, we assume a packet-switched network with 250 users. In this model, users share the total bandwidth and only use it when they have data to send.

#### c) Probability of a user transmitting and m users transmitting simultaneously

This scenario can be modeled using a Binomial Distribution because each of the 250 users is an independent "trial" with two outcomes: either they are transmitting or they are not.

- Probability a given user is transmitting (p): The user transmits 33% of the time, so p=0.33.

- Total number of users (n): n=250.

The probability that exactly m users are transmitting simultaneously is given by the binomial probability formula:

P(X=m)=(mn​)pm(1−p)n−m

Substituting our values:

P(X=m)=(m250​)(0.33)m(0.67)250−m

This formula gives the probability for any given number m of simultaneous users.

#### d) Probability of 34 or more users transmitting

We want to find the probability that the number of users transmitting, X, is 34 or more. This can be written as P(X≥34).

Calculating this directly (P(X=34)+P(X=35)+⋯+P(X=250)) is very tedious. It's much easier to calculate the complement probability and subtract it from 1.

The complement of "34 or more users" is "33 or fewer users" (P(X≤33)).

P(X≥34)=1−P(X≤33)

P(X≥34)=1−m=0∑33​(m250​)(0.33)m(0.67)250−m

Calculating this sum is computationally intensive and typically done with statistical software. The result of this calculation is approximately:

P(X≥34)≈1−1.15×10−13≈0.999...

The probability is extremely close to 1. This shows that in a packet-switched network with this many users, it's almost certain that the number of people trying to send data will be far below the number that would saturate the link (which is 34 users, from part a). This is the core benefit of statistical multiplexing in packet switching.

### Q2. Packet Delay Calculation

This question analyzes the total time it takes for data to travel across two links in a network. We are told to ignore queuing and processing delays. The total delay will therefore be the sum of transmission delay and propagation delay.

- Transmission Delay (dtrans​): Time to push all the packet's bits onto the link. dtrans​=Bandwidth (R)Packet Size (L)​

- Propagation Delay (dprop​): Time for one bit to travel across the physical link. dprop​=Propagation Speed (s)Distance (D)​

The total delay for one link is dtrans​+dprop​. Since the data must cross two identical links (A-B and B-C) and the network uses store-and-forward switching, the router at B must receive the entire packet before it can start sending it to C.

#### a) Data sent in one packet

- User Data: 20P bits

- Header: P bits

- Total Packet Size (L): 20P + P = 21P bits

- Delay for link A-B: (R21P​)+(sD​)

- Delay for link B-C: (R21P​)+(sD​)

The total time is the time for the packet to arrive at B, plus the time for it to travel from B to C.

Total Delay=(R21P​+sD​)+(R21P​+sD​)=2(R21P​+sD​)

#### b) Data sent in two equal sized packets

The 20P bits of user data are split into two packets.

- User Data per packet: 10P bits

- Header per packet: P bits

- Total size of each packet (L'): 10P + P = 11P bits

This scenario benefits from pipelining. As soon as the first packet is sent from A to B, the second packet can begin its transmission from A to B. Similarly, as soon as the first packet arrives at B, it can be forwarded to C while the second packet is still on its way to B.

Let's trace the last bit of the last packet:

- Time to transmit both packets from A: Node A finishes transmitting the second packet at time 2×dtrans​=2×R11P​.

- Arrival of the second packet at B: This last bit arrives at node B after one propagation delay. Time of arrival at B is (2×R11P​)+sD​.

- Transmission of the second packet from B to C: Node B must wait for the entire second packet to arrive before it can start sending it. It then takes one more transmission delay to send it. Time the last bit leaves B is (2×R11P​)+sD​+(R11P​)=(3×R11P​)+sD​.

- Final arrival at C: The last bit arrives at C after a final propagation delay.

Total Delay=(3×R11P​)+(2×sD​)

This can be thought of as: It takes 2 transmission times to get all the data to the last router, plus 1 more transmission time to get the last packet out of the last router. It also takes 2 propagation delays for the data to cross the two links.

### Q3. Queuing Delay

Queuing delay is the time a packet spends waiting in a buffer (queue) before it can be transmitted on the outbound link.

#### Specific Case

- Packet Size (L): 1,500 bytes

- Link Rate (R): 2 Mbps

- Situation: Our packet arrives to find:

  - 1 packet is halfway through transmission.

  - 4 other packets are already waiting in the queue.

Our packet has to wait for all the data ahead of it to be transmitted.

- Time to finish the current packet: Since it's halfway done, half of its bits remain.

  - Time remaining = 21​×Transmission Time=21​×RL​

- Time to transmit the 4 packets in the queue: Each of these packets must be transmitted fully.

  - Time for queue = 4×RL​

The total queuing delay for our packet is the sum of these times.

dqueue​=(21​×RL​)+(4×RL​)=4.5×RL​

#### General Case

- Packet Size: L

- Link Rate: R

- Situation:

  - x bits of the current packet have been transmitted. This means L-x bits remain.

  - n packets are already in the queue.

- Time to finish the current packet: The time to transmit the remaining L-x bits.

  - Time remaining = RL−x​

- Time to transmit the n packets in the queue:

  - Time for queue = n×RL​

The general formula for queuing delay in this scenario is:

dqueue​=RL−x​+RnL​=R(n+1)L−x​

### Q4. Nodal Delay Calculation

Nodal delay is the total delay experienced by a packet at a single router (or "node"). It's the sum of four components:

dnodal​=dproc​+dqueue​+dtrans​+dprop​

Let's calculate each component, making sure to use consistent units (seconds and bits).

- Given Values:

  - dproc​=5 microseconds=5×10−6 s

  - dqueue​=2 ms=2×10−3 s

  - Link Rate (R) = 10 Mbps = 10×106 bps

  - Packet Size (L) = 1,200 bytes = 1,200×8=9,600 bits

  - Link Length (D) = 400 km = 400×103 m

  - Propagation Speed (s) = 2×108 m/s

- Calculate Transmission Delay (dtrans​):

dtrans​=RL​=10×106 bps9,600 bits​=0.00096 s

- Calculate Propagation Delay (dprop​):

dprop​=sD​=2×108 m/s400×103 m​=0.002 s

- Compute Total Nodal Delay:

dnodal​=(5×10−6)+(2×10−3)+(0.00096)+(0.002) s

dnodal​=0.000005+0.002+0.00096+0.002=0.004965 s

The question asks for the answer in milliseconds (ms).

0.004965 s=4.965 ms

### Q5. End-to-End Delay

This question asks for the total time from when the first bit of a packet leaves the source until the last bit arrives at the destination. The packet traverses 3 hops (3 links). We are told to ignore propagation and processing delays.

- Packet Size (L): 100,000 bits

- Link 1 Rate (R1): 10 Mbps = 107 bps

- Link 2 Rate (R2): 2 Mbps = 2×106 bps

- Link 3 Rate (R3): 5 Mbps = 5×106 bps

Since this is a store-and-forward network, each router must receive the entire packet before forwarding it. With zero propagation delay, the end-to-end delay is simply the sum of the transmission delays on each link.

- Transmission Delay on Link 1 (dtrans1​):

dtrans1​=R1L​=10×106100,000​=0.01 s

- Transmission Delay on Link 2 (dtrans2​):

dtrans2​=R2L​=2×106100,000​=0.05 s

- Transmission Delay on Link 3 (dtrans3​):

dtrans3​=R3L​=5×106100,000​=0.02 s

The total end-to-end delay is the time the packet spends being transmitted onto each link sequentially.

Total Delay=dtrans1​+dtrans2​+dtrans3​

Total Delay=0.01+0.05+0.02=0.08 s or 80 ms

### Q6. Link Characteristics

This question explores physical properties of a data link.

#### a) Physical length of one bit

The physical length of a bit on the wire is how much space it occupies while propagating. It's the distance the bit travels in the time it takes to transmit it.

- Bit Duration: 5×10−7 s

- Propagation Speed: 2×108 m/s

Length of bit=Propagation Speed×Bit Duration

Length of bit=(2×108 m/s)×(5×10−7 s)=10×101 m=100 meters

#### b) Bandwidth-delay product

The bandwidth-delay product tells us how many bits can "fill the pipe" – i.e., how many bits can be transmitted before the first bit reaches the other end.

BDP=Bandwidth×Propagation Delay

First, we need to find the bandwidth and the propagation delay from the given information.

- Calculate Bandwidth (R): Bandwidth is the reciprocal of the bit duration.

R=Bit Duration1​=5×10−7 s1​=0.2×107 bps=2,000,000 bps=2 Mbps

- Calculate Propagation Delay (dprop​):

  - Distance = 300 km = 3×105 m

  - Propagation Speed = 2×108 m/s

dprop​=SpeedDistance​=2×108 m/s3×105 m​=1.5×10−3 s

- Calculate Bandwidth-Delay Product (BDP):

BDP=(2×106 bps)×(1.5×10−3 s)=3×103 bits=3,000 bits

### Q7. Signal-to-Noise Ratio (SNR)

This question involves calculating the SNR in both linear and decibel scales.

- Signal Power (S): 10 mW = 10×10−3 W

- Noise Power (N): 1 µW = 1×10−6 W

#### Linear SNR

The linear SNR is the direct ratio of the signal power to the noise power.

SNR=NS​=1×10−6 W10×10−3 W​=10×103=10,000

This is a unitless ratio.

#### SNR in Decibels (dB)

The formula to convert a power ratio to decibels is:

SNRdB​=10log10​(SNR)

SNRdB​=10log10​(10,000)=10log10​(104)=10×4=40 dB

### Q8. Shannon Capacity and Nyquist Theorem

This question relates the theoretical maximum data rate of a channel (Shannon) to the rate achievable with a given number of signal levels (Nyquist).

- Bandwidth (B): 3 kHz = 3000 Hz

- Measured SNRdB: 35 dB

#### a) Compute Shannon capacity

The Shannon-Hartley theorem gives the maximum theoretical data rate (capacity, C) for a noisy channel.

C=Blog2​(1+SNR)

First, we must convert the SNR from dB to a linear ratio.

35 dB=10log10​(SNR)

3.5=log10​(SNR)

SNR=103.5≈3162.28

Now, plug this into the Shannon formula:

C=3000×log2​(1+3162.28)=3000×log2​(3163.28)

Using the change of base formula for logarithms, log2​(x)=ln(2)ln(x)​:

C≈3000×0.6938.059​≈3000×11.625≈34,875 bps

The Shannon capacity is approximately 34.88 kbps.

#### b) Number of signal levels using Nyquist

The Nyquist formula relates bit rate to the number of signal levels (L) for a noiseless channel.

Bit Rate=2×B×log2​(L)

We are given a target bit rate of 30 kbps (30,000 bps).

30,000=2×3000×log2​(L)

30,000=6000×log2​(L)

6,00030,000​=log2​(L)

5=log2​(L)

L=25=32

You would need 32 signal levels to achieve a bit rate of 30 kbps.

#### c) Is this choice compatible with Shannon?

Yes, it is compatible. The Shannon capacity (≈34.88 kbps) represents the absolute theoretical maximum data rate possible on this channel, no matter how sophisticated your encoding scheme is. Our target bit rate is 30 kbps.

Since 30 kbps < 34.88 kbps, the desired rate is below the channel's theoretical limit and is therefore achievable.

### Q9 & Q14. OSI Layer Devices

These two questions are identical. Here's the breakdown of where switches, routers, and gateways operate.

#### Switches

- OSI Layer: Layer 2 (Data Link Layer).

- Role and Function: A switch's primary job is to forward frames within a single local area network (LAN). It uses the physical or MAC address of devices to make decisions. When a frame arrives, the switch looks at the destination MAC address and forwards the frame only to the port connected to that specific device. This creates separate collision domains for each port, significantly improving network efficiency compared to older hubs.

#### Routers

- OSI Layer: Layer 3 (Network Layer).

- Role and Function: A router's job is to connect different networks together and forward packets between them. It uses logical or IP addresses to make routing decisions. Routers maintain routing tables that tell them the best path to forward a packet to reach its final destination network. They are essential for the internet, connecting your home network to your ISP's network, and so on. They create broadcast domains.

#### Gateways

- OSI Layer: Can operate on all layers, but often functions at Layer 4 (Transport) through Layer 7 (Application Layer).

- Role and Function: A gateway is a general term for a device that acts as a translator between two different networks that use different protocols. While a router that connects an IP network to another IP network is a simple gateway, the term usually refers to more complex devices. For example, an email gateway might translate between the internet's SMTP protocol and an internal X.400 mail system. A proxy server is another type of gateway that can translate application-layer requests. Its key function is protocol conversion.

### Q10. Error Handling in OSI Layers

Error detection and correction are crucial for reliable communication and are handled at multiple layers for different reasons.

#### Layer 2: Data Link Layer

- Scope: Handles errors on a hop-by-hop basis (i.e., across a single physical link, like from your PC to a router).

- Why here? Physical links can be noisy (e.g., Wi-Fi interference, electrical noise on a cable). It's efficient to detect and discard corrupted frames immediately rather than letting them travel all the way across the internet.

- Mechanisms:

  - Error Detection: The most common method is the Cyclic Redundancy Check (CRC). The sender calculates a checksum value based on the frame's data and appends it to the frame's trailer. The receiver performs the same calculation. If the results don't match, the frame is considered corrupt and is dropped.

  - Error Correction: On unreliable links like wireless, protocols might include Automatic Repeat Request (ARQ), where the receiver can request a retransmission of a corrupt or lost frame.

#### Layer 4: Transport Layer

- Scope: Handles errors on an end-to-end basis (i.e., from the source application to the destination application).

- Why here? Errors can occur even if a frame is successfully transmitted across a single link. For example, a bit could flip in a router's memory. Layer 2 checks wouldn't catch this because the frame is rebuilt at each hop. The Transport Layer ensures the integrity of the entire message from start to finish.

- Mechanisms:

  - Error Detection: Protocols like TCP and UDP use a simple checksum in their headers. This checksum covers the header and the data payload. If the receiver calculates a different checksum, the entire segment is discarded.

  - Error Correction/Reliability: This is a key feature of TCP. It uses sequence numbers and acknowledgements (ACKs). The receiver sends ACKs for data it receives correctly. If the sender doesn't get an ACK within a certain time, it assumes the segment was lost and retransmits it, ensuring reliable delivery.

### Q11. OSI Model Encapsulation and De-encapsulation

This process describes how data is prepared for transmission by adding headers at each layer on the sending side (encapsulation) and how those headers are removed on the receiving side (de-encapsulation).

#### Encapsulation (Host A sending a message)

Imagine you're sending an email. The data flows down the OSI stack.

- Layer 7, 6, 5 (Application, Presentation, Session): Your email data is created and formatted. It's simply referred to as Data.

- Layer 4 (Transport): The data is broken into smaller pieces. A TCP header is added (containing source/destination port numbers). The resulting unit is called a Segment.

- Layer 3 (Network): An IP header is added to the segment (containing source/destination IP addresses). This new unit is called a Packet.

- Layer 2 (Data Link): A MAC header and trailer are added to the packet (containing source/destination MAC addresses and a CRC for error checking). This is now a Frame.

- Layer 1 (Physical): The frame is converted into a stream of bits, which are then transmitted over the physical medium (e.g., as electrical signals on an Ethernet cable or radio waves for Wi-Fi).

#### De-encapsulation (Host B receiving the message)

The bits arrive at Host B and travel up the stack.

- Layer 1 (Physical): The signals are received and converted back into a bitstream, reassembling the frame.

- Layer 2 (Data Link): The MAC header and trailer are examined. If the destination MAC address matches Host B and the frame is not corrupt (CRC check passes), the header/trailer are stripped off, and the enclosed packet is passed up to Layer 3.

- Layer 3 (Network): The IP header is examined. If the destination IP address is correct, the IP header is stripped off, and the enclosed segment is passed up to Layer 4.

- Layer 4 (Transport): The TCP header is examined. The data from the segment is reassembled in the correct order. The enclosed data is passed up to the higher layers.

- Layer 5, 6, 7 (Session, Presentation, Application): The data is processed and finally delivered to the email application on Host B, which can now display the message.

### Q12. OSI Model vs. TCP/IP Model

The OSI model is a theoretical reference model, while the TCP/IP model is a practical model that describes the protocols used in the real-world internet. Here are three major differences.

#### 1. Number of Layers and Complexity

- OSI Model: Has 7 layers (Application, Presentation, Session, Transport, Network, Data Link, Physical). It is very prescriptive and makes a clear distinction between the roles of each layer.

- TCP/IP Model: Typically described with 4 layers (Application, Transport, Internet, Network Access/Link). It combines the functions of OSI's Application, Presentation, and Session layers into its single Application layer. Similarly, it combines the Data Link and Physical layers into its Network Access layer. It is more descriptive of how the internet actually works.

#### 2. Origin and Purpose

- OSI Model: Developed by the International Organization for Standardization (ISO). It is a generic, protocol-independent standard designed before the protocols were implemented. Its purpose was to be a universal guide for building networks.

- TCP/IP Model: Developed by the US Department of Defense. It is a protocol-dependent model that was created to describe the protocols (like TCP and IP) that were already in use on the ARPANET (the precursor to the internet).

#### 3. Service Type Distinction

- OSI Model: Makes a very clear distinction between the concepts of service, interface, and protocol. It also clearly defines both connection-oriented and connectionless communication in both the Network Layer and the Transport Layer.

- TCP/IP Model: Doesn't rigidly separate these concepts. It primarily supports connectionless communication at the Internet (Network) Layer (via IP) and supports both connection-oriented (TCP) and connectionless (UDP) at the Transport Layer.

### Q13. Functions of the Seven OSI Layers

Here are the main functions of each layer, from top to bottom, with a real-world example.

- Layer 7: Application Layer

  - Function: Provides the interface for network services to the end-user's applications. It's what the user interacts with.

  - Real-world Example: Your web browser using the HTTP protocol to request a webpage. Other examples include FTP (file transfer) and SMTP (email).

- Layer 6: Presentation Layer

  - Function: Responsible for data translation, character encoding, compression, and encryption/decryption. It ensures that data sent from one system can be read by another.

  - Real-world Example: TLS/SSL encryption that turns HTTP into HTTPS. Also, formatting data as a JPEG image or ASCII text.

- Layer 5: Session Layer

  - Function: Manages, maintains, and terminates connections (or "sessions") between applications. It handles authentication and authorization.

  - Real-world Example: When you log in to a banking website, the Session Layer establishes a secure session that stays active while you navigate the site and logs you out after a period of inactivity.

- Layer 4: Transport Layer

  - Function: Provides end-to-end data transfer services. It handles segmentation, reassembly, flow control, and error correction to ensure reliable communication.

  - Real-world Example: The TCP protocol breaking a large file into segments, ensuring each one arrives at the destination, and re-requesting any that get lost.

- Layer 3: Network Layer

  - Function: Responsible for logical addressing (like IP addresses), path determination, and routing of packets across multiple networks.

  - Real-world Example: A router using the destination IP address on a packet to decide which of its connections is the best next hop to send the packet on its journey across the internet.

- Layer 2: Data Link Layer

  - Function: Handles framing (packaging bits into frames), physical addressing (MAC addresses), and error detection for a single physical link.

  - Real-world Example: Your computer's Ethernet card or Wi-Fi adapter creating a frame and sending it to your local router using the router's MAC address.

- Layer 1: Physical Layer

  - Function: Defines the physical and electrical specifications for the connection. It is responsible for transmitting raw bits over a physical medium.

  - Real-world Example: The electrical signals traveling over the copper wires in an Ethernet cable, or the light pulses in a fiber-optic cable.